| Product Overview |  |
| --- | --- |
| The S9055-5G-WIFI7 is a new, high-performance WiFi 7-enabled 5G CPE brought to market by Ricon Mobile Inc. Using the latest Samsung chipset for its cellular solution, the product features a WiFi architecture based on the 802.11be standard and houses a 2x2+2x2 entry-level dual-band WiFi configuration. It combines strong cellular performance with a high cost advantage. The product is fully compliant with the 3GPP R16 protocol standard; besides supporting different network types including NR-SA, NR-NSA, FDD-LTE and TDD-LTE, it offers strong carrier aggregation capabilities. Essentially, by converting cellular network access into WiFi and wired connections, it provides network access for various end devices such as smartphones, tablets and personal computers, and can support numerous service types. With its plug-and-play architecture, this device requires no extra setup and has a wide range of uses across different scenarios such as homes, street-front shops, and small and micro businesses. |  |

.

| Product Features |  |
| --- | --- |
| High-Speed Internet Experience | This device supports the common frequency bands used worldwide, is compatible with both 5G SA and NSA networks, and supports bandwidths of up to 200M on 5G networks, delivering an excellent 5G internet experience to users. Under an SA network, the device supports 5CC carrier aggregation; under an NSA network, it supports a rich set of ENDC combinations, delivering maximum download and upload speeds. Under an LTE network, the device can support up to 6CC downlink carrier aggregation and 2CC uplink carrier aggregation, delivering outstanding uplink and downlink speeds even on a 4G network. |
| Latest WiFi 7 Standard, Dual-Band Concurrent Connection | This device supports the latest WiFi 7 standard, offers 2.4GHz and 5GHz dual-band support, supports 2x2 MIMO technology with 160M ultra-wide bandwidth, and reaches theoretical speeds of up to 3600Mbps, delivering an ultra-high-speed WiFi access experience to users. The device uses OFDMA technology, which allows it to transmit data packets to multiple terminal devices simultaneously; combined with MU-MIMO technology, it supports multiple terminal devices accessing and using services at the same time. The latest Multi-Link Operation technology allows a single STA device to connect to the CPE over multiple bands simultaneously, which greatly increases throughput and reduces latency. |
| Multiple Network Options and Multi-Service Support | The product supports mobile cellular and/or wired broadband as uplink, and also supports WiFi and/or LAN ports for terminal device access. Network setup is much more flexible. The device is also equipped with an RJ11 voice interface. When paired with a phone, it can support voice call services over VoLTE/VoNR. Multiple services such as internet access, video streaming and voice calls can be carried out simultaneously, meeting a wide range of user needs. |
| High Availability with Dual-Link Uplink | - The product supports configuring both cellular and wired uplink connections at the same time. Users can set link priority and detection criteria. When the primary link is disconnected, the device can automatically switch to the backup link, which significantly increases the device's high availability. - The device supports self-healing. In the event of a cellular network fault causing a network outage, the device automatically reconnects and restores the network connection once the network recovers. |
| Device Protection Against Theft and Misuse | The device can be customized for use with specific PLMNs, thereby protecting operators' rights and interests. Supports 5G frequency band and cell locking functions. The device becomes unusable when taken outside its designated deployment area, which protects the equipment and prevents it from being used for other purposes. The device supports different administrator accounts and authorization levels that can be customized according to operator requirements. |

| Hardware Features |  |
| --- | --- |
| CPU | Realtek Dual-core ARM Cortex-A55 @1.2 GHz and Tri-core RISC-V @800 MHz for WiFi Offload |
| Memory | DDR: 512MB Nand Flash: 128MB |
| Cellular Chipset | Samsung S5300 |
| WiFi Chipset | Realtek |

| Cellular Features |  |
| --- | --- |
| 3GPP Compliance | Rel-16 |
| Supported Technology | 5G NR NSA & SA: Option 3x/3a/3 and Option 2 4G LTE: TDD & FDD 3G UMTS: WCDMA |
| UE Category | LTE DL Cat 20, UL Cat 18 |
| MIMO | NR: DL 4x4, UL 2x2 LTE: DL 4x4 |
| Carrier Aggregation | NR CA: DL 5CC 200M; UL 2CC 200M LTE CA:DL 6CC 120M; UL 2CC 40M |
| Theoretical Peak Rate | NR SA: 4.67Gbps DL/1.25Gbps UL NR NSA: 6.47Gbps DL/730Mbps UL LTE: 2.0Gbps DL/211Mbps UL |
| Cellular Antennas | 4 internal Omni-Directional antennas |
| SIM Support | 1 * nano SIM 1 * eSIM (Optional) |

| WiFi Features |  |
| --- | --- |
| WiFi Protocol | 2.4 GHz: IEEE 802.11 b/g/n/ax/be 5 GHz: IEEE 802.11 a/n/ac/ax/be |
| MIMO | 2.4 GHz: 2x2 MIMO 5 GHz: 2x2 MIMO |
| WiFi Bandwidth | 2.4 GHz: 20/40 MHz 5 GHz: 20/40/80/160 MHz |
| Theoretical Throughput | Total theoretical WiFi throughput: 3600Mbps 2.4GHz: 688Mbps (2ss 40MHz) 5GHz: 3600Mbps (2ss 160MHz) |
| WiFi Antennas | 2.4 GHz: 2 internal Omni-Directional antennas 5 GHz: 2 internal Omni-Directional antennas |

| Interfaces, Buttons and Indicator Features |  |
| --- | --- |
| WAN Port | 1 * 2.5GE port,100M/1000M/2500M adaptive, type:RJ45 |
| LAN Port | 3 * GE ports,100M/1000M adaptive,type:RJ45 |
| Voice Port | 1 * FXS VoLTE/VoNR port (optional), type: RJ11 |
| USB Port | 1* USB2.0 |
| Power Supply Port | 1 * DC-Jack port |
| Other Port | 2 * TS9 ports, for external 5G antenna connection |
| pSIM Interface | 1 * SIM slot 4FF (nano SIM) |
| Indicators | 1 * 5G (Green / Yellow / Red) 1 * 4G (Green / Yellow / Red) 1 * WiFi (Green) 1 * Phone or Internet (Green) 1 * System (Green) 1 * Power (Green) |
| Buttons & Pinhole | 1 * Power On/Off button 1 * Reset pinhole 1 * WPS button |

| Physical Features |  |
| --- | --- |
| Dimensions | 95mm*114mm*186mm (L*W*H) |
| Weight | 700g |
| Material | ABS Housing IP30 |
| Color | White |
| Installation | Tabletop |

| Power Module |  |
| --- | --- |
| Power Supply | Power adapter (Input: 100~240V, 50/60Hz) |
| Output Voltage | DC 12V |
| Output Current | DC 2A |
| Power Consumption | Max 24W |

| Environmental Parameters |  |
| --- | --- |
| Operating Temperature | -10℃ + 45℃ |
| Storage Temperature | -20℃ + 70℃ |
| Operating Humidity | %5 - %95 (non-condensing) |
| Storage Humidity | %5 - %95 (Non-condensing) |
| Protection Class | IP30 |

| Software Features |  |
| --- | --- |
| Status Management | System Status / Device Information Mobile Network Status WAN Status LAN Status WLAN Status TR069 Status Web GUI CLI / Telnet / SSH SNMPv2 SIM Lock, Band Lock, Operator Lock Traffic Statistics |
| Mobile Network | APN Management PIN Code Management Network Mode Selection Mobile Data Switch & Roaming Switch Short Message Service Band Lock Cell Lock VoLTE/ VoNR Mobile Network Self-Recovery |
| WAN Configuration | Operating Mode: Routing mode, Bridge mode IPv4, IPv6, IPv4/IPv6 PPPoE, DHCP Client/Server DNS, Dynamic DNS Uplink WAN mode: Cellular, Wired, Link-backup |
| LAN Configuration | DDHCP Client/Server NAT/NAPT ALG Virtual Server (Port Forwarding) MAC Management IGMP Snooping* IGMP Proxy* |
| WiFi Configuration | Basic Settings (SSID, password, etc.) Advanced Settings (protocol, BW, channel, etc.) Cipher Mode (OPEN/WPA/WPA2/WPA3) WiFi Protected Set-up Multiple SSID Band Steering Guest WiFi STA Management Mesh Settings |
| Routing | Static routing v4/v6 |
| VPN Protocol | L2TP/PPTP IPSec VxLAN |
| Security & Firewall | Basic firewall Anti-attack (ARP, DDoS, etc.) Anti-scan Port filtering MAC filtering DMZ |
| Additional Functions | UPnP Parental Control |
| Device Management | Cloud Management Platform (Optional) Language Management User Management Firmware Update & Rollback Remote Management: TR069/XMPP*/SNMP FOTA Reboot, Factory Reset NTP Management Configuration Export and Import Diagnostics |

Application Topology

| Model Information |  |
| --- | --- |
| Model | S9055-5G-WIFI7 |
| Chipset | Samsung S5300 |
| Applicable Region | EMEA Asia Pacific |
| Supported Bands | 5G NR:n1/n2/n3/n5/n7/n8/n20/n26/n28/n38/n40/n41/n66/n67/n71/n75/ n76/n77/n78 LTE-FDD:B1/B2/B3/B4/B5/B7/B8/B20/B26/B28/B32/B66/B67/B71 LTE-TDD:B38/B40/B41/B42/B43/B46/B48 WCDMA:B1/B2/B3/B5/B8 |

Product Views

| Revision History |  |
| --- | --- |
| Version | V.0.1.02 |
| Date | August 2026 |
| Description | Preliminary version prepared for product description only |

About Ricon

At Ricon, we create technologies that make communication easier.

As a 24-year-old B2B technology innovation company, we design networks that sense, think and act across mobile, fixed and cloud networks.

Service providers, businesses and partners around the world trust Ricon today to deliver secure, reliable and sustainable networks, and work with us to create the digital services and applications of the future.

© 2026 Ricon

| Ricon Canada Office (HQ) 5063 North Service Road Burlington Ontario L7L 5H6 Canada riconrequest@riconmobile.com +1 (905) 921-3122 +1 (905) 336-2450 | Ricon Turkey Office Ahi Evran Cad. No: 21 Polaris Plaza Floor: 6-8 Maslak, Istanbul riconturkiye@riconmobile.com +90 212 346 26 00 | Ricon Turkey Office ITU Reşitpaşa Mah. ITU ARI 3 Teknokent No:4-Z101 Sarıyer, Istanbul riconturkiye@riconmobile.com +90 212 346 26 00 | Ricon Italy Office Ricon Italy Office Ricon Mobile Italy Srl Via Gaetano Negri 8 20123 Milano Italy riconitaly@riconmobile.com +39 335 195 06 86 |
| --- | --- | --- | --- |