S9980DSL-LTE

| Features LTE / 4G / 3G support Failover/Failback IPSec VPN OpenVPN WireGuard GRE Tunneling Firewall MAC / IP Filter DoS/DDoS Protection HTTPS / SSH Connection Static & Dynamic Routing BGP VRF / VLAN NAT / Port Routing / DMZ DHCP Server / Client Bridging / Passthrough Policy-based Routing (PBR) Web GUI (HTTPS) CLI (SSH / Telnet) OTA Software Update Cloud Management Platform (Optional) WPA2 / WPA3 MAC Filter WPS / Wi-Fi Scheduling Gigabit Ethernet ports Connection timers Task definition via scripts | Technical specifications Dimensions and weight - Height: 40mm - Width: 230mm - Depth: 150mm - Weight: 784g Environmental - Idle: 80mA + 12VDC - Single Module Power Consumption: <7W - IP54 Protection - Operating Temperature −30…+70 °C - Storage Temperature −30…+85 °C - Humidity <%95 (non-condensing) Certifications - Industry Canada, PTCRB, CE, FCC, UL, E57, RoHS - EN 62311 - EN 60950- 1+A11+A1+A12 - EN 301-489-1/-7/-17/-24, EN 55022, EN 55024 - EN 301 511, EN 301 908-1/-2, EN 300 328 |
| --- | --- |

| General features |  |
| --- | --- |
| CPU | - RTL8685PB MIPS 32Bit |
| Memory | 64MB |
| HW Watchdog | - Connection timers - Ping / Uptime Watchdog - Script tasks |
| SPI Flash | - 128MB |
| Module | - Quectel EC25-EC Module |
| Interfaces and Configuration |  |
| LAN | 4x1000 Mbps |
| WAN | 1× 10/100/1000 Base-T RJ45 |
| DHCP Server | DHCP Server/Client/Relay, DNS |

| Cellular Features |  |
| --- | --- |
| Cellular Modem Module | EC25-EC |
| Module Type | MINI PCIe |
| Region | Global |
| Chipset | Qualcomm MDM9207-1 |
| Category | LTE: Cat.4 |
| Cellular Bands (SDX62) | - 4G LTE: B1 / B3 / B7 / B8 / B20 / B28A - 3G UMTS/WCDMA: B1 / B8 - 2G GSM: B3 / B8 |
| SIM Support | - (U)SIM |

| Security Features |  |
| --- | --- |
| Firewall | Identity-Based Firewall Identity-Based Firewall DoS |
| Filters | Access restriction Restrict wireless access |
| Misc | Security logging Packet capture logging (pcap) |

| Management Features |  |
| --- | --- |
| Management Interfaces | Web GUI (HTTPS) CLI (SSH / Telnet) SNMP Syslog / Event Log OTA Firmware Update Cloud Management Platform SSH SNMPv2 TR-069 |
| Version Management | Image Checksum Active version and backup version Automatic rollback feature when remote upgrade fails |
| Account Management | TACACS/TACACS+, Radius Auth. |
| Retrievable Parameters WEB UI / CLI / TR-069 / SNMP | CLI command line access SNMP remote monitoring and management Web-based graphical interface management |

| Cellular Modem Features |  |
| --- | --- |
| APN Data Services | Dynamic APN support SIM PIN management Bridge Mode Support |
| Modem Interface Configuration | SIM slot selection Full band scan Modem Band Lock Automatic/On-demand dialing Radio Enable/Disable Pppd and DHCP dial options |
| Modem Backup | Via interface |

| Wi-Fi |  |
| --- | --- |
| Chipset | - Realtek RTL8812FR |
| Wi-Fi Features | - Wi-Fi Radio: 2.4 GHz, up to 80 MHz - Wi-Fi Standards: IEEE 802.11 a/b/g/n/ac (Wi-Fi 4 and Wi-Fi 5) - Channel Bandwidth Support: 20/40/80 MHz - 2.4 GHz Maximum Data Rate: 300 Mbps (2×2 MIMO, 40 MHz) |
| MIMO | - 2.4 GHz: 2×2 MIMO |
| Region | - Global |
| Wireless Security | - Protocols: WEP, WPA, WPA2, WPA3 - Encryption: AES, TKIP |
| Mode | - Access Point (AP) and Wi-Fi Client (Station/Client) modes are supported |

| Network Features |  |
| --- | --- |
| Routing | Static Routing, Dynamic Routing BGP OSPF / RIP VLAN NAT / Port Forwarding / DMZ DHCP Server / Client Bridging / Passthrough Policy-Based Routing (PBR) |
| Tunnels | IPSec VPN WireGuard OpenVPN L2TP PPTP MPLS GRE Tunneling Firewall MAC / IP Filtering DoS Protection HTTPS / SSH Access |
| Routing | NAT Support Port Forwarding DMZ Support |
| Load Balancing | Per-Packet Routing Support |
| Ordering Information and Accessories |  |
| S9980DSL-LTE | CAT4 Cellular Radio Interface 4xLAN Gigabit, 1xWAN Gigabit, SIM Slot xDSL 2.4 GHz, 5GHz Wi-Fi 802.11b/g/n/ac |
| Defaults |  |
| Box Contents | S9980DSL-LTE 2 x 3m LTE antenna 2 x 2.4 GHz, 5GHz WiFi antenna 1m CAT6 cable Mounting kit 12 VDC 1.5 A Power Adapter |

About Ricon

At Ricon, we create technologies that make communication easier.

As a 24-year-old B2B technology innovation company, we design networks that sense, think and act across mobile, fixed and cloud networks.

Service providers, businesses and partners around the world trust Ricon today to deliver secure, reliable and sustainable networks, and work with us to create the digital services and applications of the future.

© 2026 Ricon

| Ricon Canada Office (HQ) 5063 North Service Road Burlington Ontario L7L 5H6 Canada riconrequest@riconmobile.com +1 (905) 921-3122 +1 (905) 336-2450 | Ricon Turkey Office Ahi Evran Cad. No: 21 Polaris Plaza Floor: 6-8 Maslak, Istanbul riconturkiye@riconmobile.com +90 212 346 26 00 | Ricon Turkey Office ITU Reşitpaşa Mah. ITU ARI 3 Teknokent No:4-Z101 Sarıyer, Istanbul riconturkiye@riconmobile.com +90 212 346 26 00 | Ricon Italy Office Ricon Italy Office Ricon Mobile Italy Srl Via Gaetano Negri 8 20123 Milano Italy riconitaly@riconmobile.com +39 335 195 06 86 |
| --- | --- | --- | --- |